| Course Name: | Elements of Electrical and Electronics Engineering | Semester: | I/II |
| --- | --- | --- | --- |
| Date of Performance: |  | Batch No: | C1 |
| Faculty Name: |  | Roll No: | 16010121152 |
| Faculty Sign & Date: |  | Grade/Marks: | / 25 |

Tejas Pundlik – C1 – 16010121152

Experiment No: 9

Title: Measurement of Power using Two Wattmeter Method

Aim and Objective of the Experiment:

- To measure the power of three phase power using Two Wattmeter Method

COs to be achieved:

CO1: Analyze resistive networks excited by DC sources using various network theorems.

Circuit Diagram/ Block Diagram:

Circuit Diagram

Stepwise-Procedure:

- 1. Connect the circuit as shown in circuit diagram - 2. Increase the load and note down the reading VL,IL,W1 and W2 - 3. Practically you will obtain total power W=W1+W2 - 4. Theoretically power is measured by using formula P=√3VLILcosϕ, using cosϕ=1(unity) for resistive load.

Observation Table: STAR LOAD Sr No.; Load; VL; IL; W1; W2; W=W1+W2; √3VLILcosϕ. 1; R = 100; 398 V; 2.28 A; 791 W; 793 W; 1584 W; 1571.68 W. 2; R = 100 L = 1H; 398 V; 0.70 A; -59.4 W; 205 W; 145.6 W; 146.36 W. 3; R = 100 C = 10uF; 398 V; 0.69 A; 201 W; 60 W; 261 W; 260.69 W. DELTA LOAD Sr No.; Load; VL; IL; W1; W2; W=W1+W2; √3VLILcosϕ. 1; R = 100; 395 V; 6.89 A; 2370 W; 2380 W; 4750 W; 4713.72 W. 2; R = 100 L = 1H; 395 V; 2.1 A; -178 W; 614 W; 436 W; 428.36 W. 3; R = 100 C = 10uF; 395 V; 2.06 A; 605 W; -179 W; 426 W; 415.78 W.

Screenshot of Output: Star Load Delta Load Theoretical Proof

Conclusion:

We have successfully measured the power of a three-phase power system using two watt-meters.

Signature of faculty in-charge with Date: