| Course Name: | Elements of Electrical and Electronics Engineering | Semester: | I/II |
| --- | --- | --- | --- |
| Date of Performance: |  | Batch No: | G3 |
| Faculty Name: | Milind Marathe | Roll No: | 16010421063 |
| Faculty Sign & Date: |  | Grade/Marks: | / 25 |

Experiment No: 10

Title: Inverting and Non-inverting amplifier using OPAMP

Aim and Objective of the Experiment:

- To understand the open loop configuration of OPAMP - To understand the concept of negative feedback and closed loop configuration of OPAMP. - To understand inverting and Non-inverting amplifier of OPAMP - To find gain of inverting and non-inverting amplifiers

COs to be achieved:

CO5: Understand operational amplifier and its applications

Circuit Diagram/ Block Diagram:

Pin diagram of IC 741 1. Inverting Amplifier 2. Non-inverting Amplifier Observation Table: 1. A. Inverting Amplifier: DC input Voltage Sr.No.; Vin (V); Vout (V); Practical Gain = Vout/Vin; Theoretical Gain=-RF/R1. 1.; 0.9; -4.49294; -4.99215555556; 5. 2.; 0.65; -3.24302; -4.98926153846; 5. 3.; 3.1; -10.4979; -3.38641935484; 5. 4.; -2.1; 10.5061; -5.0029047619; 5. In observation 3 the output will never cross 11 because of saturation as the value of VCC and VEE is 12. 1. B. Inverting Amplifier: AC input Voltage Sr.No.; Frequency (Hz); Vin(p-p) (V); Vout(p-p) (V); Practical Gain = Vout/Vin; Theoretical Gain=-RF/R1. 1.; 1K; 3; 15; 5; 5. 2.; 1K; 4; 20; 5; 5. 3.; 1K; 5; 22; 4.4; 5. 4.; 1K; 2; 10; 5; 5. In observation 3 the output will never cross 22 because of saturation as the value of VCC and VEE is 12. 2. A. Non-inverting Amplifier: DC input Voltage Sr.No.; Vin (V); Vout (V); Practical Gain = Vout/Vin; Theoretical Gain=1+RF/R1. 1.; 2; 10.9978; 5.4989; 6. 2.; 1; 6.00619; 6.00619; 6. 3.; 1.5; 9.00591; 6.00394; 6. 4.; -1.2; -7.19254; 5.99378333333; 6. 2. B. Non-inverting Amplifier: AC input Voltage Sr.No.; Frequency (Hz); Vin(p-p) (V); Vout(p-p) (V); Practical Gain = Vout/Vin; Theoretical Gain=1+RF/R1. 1.; 1K; 1; 6; 6; 6. 2.; 1K; 1.5; 9; 6; 6. 3.; 1K; 4; 22; 5.5; 6. Screenshots - Inverting AC amplifier - Saturation - Unsaturated - Inverting Amplifier DC - Unsaturated - Saturated - Non Inverting DC Amplifier - Saturated - Unsaturated - Non Inverting AC Amplifier - Unsaturated - Saturated Theoretical calculation- Non-Inverting- Inverting Post Lab Subjective/Objective type Questions: 1. List the characteristics of an Ideal operational amplifier. Ans- - Infinite input impedance - Zero output impedance - Zero common-mode gain, or, infinite common-mode rejection - Infinite open-loop gain A - Infinite bandwidth 2. List the important parameters of the IC 741 operational amplifier. Ans- Important Parameters are- - Open Loop Gain-105 to 108 - Input Resistance- 105 to 1013 - Output Resistance- 10 to 100

Conclusion:

We successfully understood the inverted as well as non inverted amplification IC 741

Signature of faculty in-charge with Date: