| 3.Course Name: | Elements of Electrical and Electronics Engineering | Semester: | I/II |
| --- | --- | --- | --- |
| Date of Performance: |  | Batch No: | B2 |
| Faculty Name: |  | Roll No: | 16010121118 |
| Faculty Sign & Date: |  | Grade/Marks: | / 25 |

Experiment No: 10

Title: Inverting and Non-inverting amplifier using OPAMP

Aim and Objective of the Experiment:

- To understand the open loop configuration of OPAMP - To understand the concept of negative feedback and closed loop configuration of OPAMP. - To understand inverting and Non-inverting amplifier of OPAMP - To find gain of inverting and non-inverting amplifiers

COs to be achieved:

CO5: Understand operational amplifier and its applications

Circuit Diagram/ Block Diagram:

Pin diagram of IC 741 1. Inverting Amplifier 2. Non-inverting Amplifier Observation Table: 1. A. Inverting Amplifier: DC input Voltage Sr.No.; Vin (V); Vout (V); Practical Gain = Vout/Vin; Theoretical Gain=-RF/R1. 1.; 1.5; 0.25; 0.17; 1/6. 2.; 1; 0.17; 0.17; 1/6. 3.; 0.5; 0.08; 0.16; 1/6. 1. B. Inverting Amplifier: AC input Voltage Sr.No.; Frequency (Hz); Vin(p-p) (V); Vout(p-p) (V); Practical Gain = Vout/Vin; Theoretical Gain=-RF/R1. 1.; 1000; 1.5; 0.25; 0.17; 1/6. 2.; 1000; 1; 0.16; 0.16; 1/6. 3.; 1000; 0.5; 0.09; 0.17; 1/6. 2. A. Non-inverting Amplifier: DC input Voltage Sr.No.; Vin (V); Vout (V); Practical Gain = Vout/Vin; Theoretical Gain=1+RF/R1. 1.; 1.5; 9.01; 6.007; 6. 2.; 1; 6.01; 6.01; 6. 3.; 0.5; 2.99; 5.98; 6. 2. B. Non-inverting Amplifier: AC input Voltage Sr.No.; Frequency (Hz); Vin(p-p) (V); Vout(p-p) (V); Practical Gain = Vout/Vin; Theoretical Gain=1+RF/R1. 1.; 1000; 1.5; 9.01; 6.07; 6. 2.; 1000; 1; 5.97; 5.97; 6. 3.; 1000; 0.5; 2.98; 5.95; 6. Screenshots:- 1.A: 1.B: 2.A: 2.B: Post Lab Subjective/Objective type Questions: 1. List the characteristics of Ideal operational amplifier. An ideal op amp is usually considered to have the following characteristics: - Infinite open-loop gain G = vout / vin - Infinite input impedance Rin, and so zero input current - Zero input offset voltage - Infinite output voltage range - Infinite bandwidth with zero phase shift and infinite slew rate - Zero output impedance Rout, and so infinite output current range - Zero noise - Infinite common-mode rejection ratio (CMRR) - Infinite power supply rejection ratio. 2. List the important parameters of IC 741 operational amplifier. We furnish the typical parameters of a 741 IC in the table below. Parameters; Typical Range; Ideal Value. Open Loop Gain(A); 105 to 108; ∝. Input Resistance(Ri); 105 to 1013; ∝. Output Resistance(Ro); 10 to 100; 0.

Conclusion:

We learnt about the features of IC 741.

Signature of faculty in-charge with Date: